Requirements Test Plan

Centralized Routing Management System

| Course | Informatics for Telecommunications |
| --- | --- |
| Program | Electronic Engineering and Telecommunications |
| Project | Centralized Routing System |
| Team name | NetCore Team |
| Team members | Isabella Perez Hoyos (Documenter + Developer + Testet) |
| Professor | Oscar Mauricio Caicedo Rendon |
| Date | April 25, 2026 |
| Version | 1.0 |

# 1. Test Plan Objective

The objective of this test plan is to verify that the centralized routing system satisfies all defined functional and non-functional requirements. The tests validate router registration, topology collection, routing algorithm execution (Dijkstra), routing table generation and distribution, Longest Prefix Matching forwarding, secure communication via TCP with TLS encryption, fault detection through heartbeat, queue management, and metrics reporting. Each test case was initially executed manually and guided. After the first successful execution, tests are automated using Python's unittest framework to ensure continuous validation throughout the development process.

# 2. Testing Scope

| Included in testing | Status |
| --- | --- |
| Controller application | Yes |
| Router application | Yes |
| Communication protocol | Yes |
| JSON message format | Yes |
| Topology storage | Yes |
| Routing algorithm (Dijkstra) | Yes |
| Routing table generation and distribution | Yes |
| Command-line interface | Yes |
| Error handling | Yes |
| Longest Prefix Matching (LPM) forwarding | Yes |
| TLS encryption and token authentication | Yes |
| Heartbeat and fault detection | Yes |
| FIFO queue and tail drop policy | Yes |
| Metrics reporting and storage | Yes |
| Test automation (Python unittest) | Yes |

Priority will be given to automated testing of core routing, forwarding, topology, and messaging behaviors. Security and monitoring tests may combine guided execution and partial automation.

# 3. Test Environment

| Element | Description |
| --- | --- |
| Operating system | Windows 11 |
| Python version | Python 3 using the project virtual environment. |
| Communication protocol | TCP sockets with TLS encryption and JSON messages. |
| Libraries used | socket, ssl, json, threading, unittest, mysql-connector-python and standard Python modules. |
| Number of router applications | 5 router instances validated in the final integration demo: R1, R2, R3, R4 and R5. |
| Controller IP address | 127.0.0.1 for local validation; configurable for distributed execution. |
| Controller port | 6000 |

# 4. Test Case Template

| Field | Description |
| --- | --- |
| Test case ID | TC-XX |
| Related requirement | FR-XX or NFR-XX |
| Test name | Short name of the test |
| Objective | What the test verifies |
| Preconditions | Conditions required before executing the test |
| Input data | Data used during the test |
| Test steps | Ordered steps to execute the test |
| Expected result | Correct system behavior |
| Actual result | Observed behavior |
| Status | Passed / Failed |
| Evidence | Screenshot, log, file, or terminal output |
| Observations | Additional comments |

# 5. Functional Test Cases

## TC-01: Router registration with the controller

| Field | Description |
| --- | --- |
| Test case ID | TC-01 |
| Related requirement | FR-01 |
| Test name | Router registration with the controller |
| Objective | Verify that a router can register successfully with the controller. |
| Preconditions | Controller application is running. |
| Input data | Router ID: R1, IP: 127.0.0.1, Port: 5001 |
| Test steps | 1. Start the controller. 2. Start router R1. 3. Send registration message: {"type": "AUTH", "id": "R1", "ip": "127.0.0.1", "port": 5001, "token": "<token_R1>"}. 4. Check controller registry in database table routers. |
| Expected result | The controller stores R1 successfully and confirms registration with AUTH_OK message. |
| Actual result | Controller received AUTH from router R1, responded with AUTH_OK, and stored the router successfully in the routers table. |
| Status | Passed |
| Evidence | Terminal output from controller and router. SQL query to routers and eventos tables confirming registration. |
| Observations | Validated through integrated execution with the running controller and router instances. This is part of the current Sprint 1 working code. Revalidated in the final Sprint 3 system. |

## TC-02: Send neighbor information to controller

| Field | Description |
| --- | --- |
| Test case ID | TC-02 |
| Related requirement | FR-02, FR-03 |
| Test name | Send neighbor information to controller |
| Objective | Verify that routers can send neighbor and link cost information and the controller builds the topology graph. |
| Preconditions | Controller and routers R1, R2, R3 are running and registered. |
| Input data | R1 neighbors: R2 cost 10, R3 cost 5 |
| Test steps | 1. Start controller. 2. Start routers R1, R2, and R3. 3. Send neighbor information from R1: {"type": "NEIGHBORS", "from": "R1", "links": [{"to": "R2", "cost": 10}, {"to": "R3", "cost": 5}]}. 4. Check topology graph stored in controller (table enlaces). |
| Expected result | The controller stores links R1-R2 (cost 10) and R1-R3 (cost 5) in the topology graph. |
| Actual result | The controller received NEIGHBORS from R1 and stored links R1-R2 (cost 10) and R1-R3 (cost 5) in the enlaces table. |
| Status | Passed |
| Evidence | Terminal output from controller and router. SQL query to enlaces and eventos tables confirming topology storage. |
| Observations | Validated with three configured router instances (R1, R2, R3). Revalidated in the final Sprint 3 system. |

## TC-03: Dijkstra shortest path calculation

| Field | Description |
| --- | --- |
| Test case ID | TC-03 |
| Related requirement | FR-04 |
| Test name | Dijkstra shortest path calculation |
| Objective | Verify that the controller computes the shortest path correctly using Dijkstra's algorithm. |
| Preconditions | Network topology is stored in the controller. |
| Input data | Topology: R1-R2 cost 2, R2-R3 cost 1, R1-R3 cost 5 |
| Test steps | 1. Load topology into controller graph. 2. Execute Dijkstra from R1. 3. Check shortest path from R1 to R3. 4. Verify cost and path sequence. |
| Expected result | The shortest path from R1 to R3 is R1 -> R2 -> R3 with total cost 3. |
| Actual result | The Dijkstra service returned the expected shortest paths over the test topology. In the final five-router topology, R1 to R5 was computed as R1 -> R3 -> R4 -> R5 with total cost 10, and R5 to R4 as R5 -> R2 -> R3 -> R4 with total cost 5. |
| Status | Passed |
| Evidence | Automated unittest execution and terminal output from path_cli. |
| Observations | Validated as a unit-level algorithm test and through the final guided demo. |

## TC-04: Generate routing table for each router

| Field | Description |
| --- | --- |
| Test case ID | TC-04 |
| Related requirement | FR-05 |
| Test name | Generate routing table for each router |
| Objective | Verify that the controller generates correct routing tables from Dijkstra results. |
| Preconditions | Topology is available and shortest paths were computed. |
| Input data | Network topology with at least four routers (R1, R2, R3, R4). |
| Test steps | 1. Load topology with 4 routers. 2. Execute Dijkstra algorithm. 3. Generate routing table for R1. 4. Verify each entry contains: destination prefix, prefix length, next hop, and cost. |
| Expected result | Routing table for R1 contains valid entries for all reachable destinations with correct next hop and cost. |
| Actual result | The controller generated routing tables for all active routers. In the final topology with R1-R5, 20 routing entries were generated and stored. |
| Status | Passed |
| Evidence | link_cost_cli output showing ROUTING_TABLES_STORED with saved_entries: 20. |
| Observations | Five routers x four destinations each produced 20 calculated routes. |

## TC-05: Send routing table to router

| Field | Description |
| --- | --- |
| Test case ID | TC-05 |
| Related requirement | FR-06 |
| Test name | Send routing table to router |
| Objective | Verify that the controller sends the correct routing table to each router via TCP+TLS+JSON. |
| Preconditions | Controller and router R1 are running. TLS connection is established. |
| Input data | Routing table for R1 with at least 3 entries. |
| Test steps | 1. Generate routing table in controller. 2. Send UPDATE_TABLE message to R1: {"type": "UPDATE_TABLE", "entries": [...]}. 3. Check that R1 receives the message. 4. Verify table is stored in R1 local memory or lightweight local persistence. |
| Expected result | Router R1 receives and stores its routing table correctly in its local database. |
| Actual result | Routers received UPDATE_TABLE messages after registration/topology exchange and stored the received forwarding tables locally. |
| Status | Passed |
| Evidence | Router terminal output showing UPDATE_TABLE received; controller terminal output showing UPDATE_TABLE sent. |
| Observations | Validated with TLS-enabled controller-router communication. |

## TC-06: Display routing table in CLI

| Field | Description |
| --- | --- |
| Test case ID | TC-06 |
| Related requirement | FR-07 |
| Test name | Display routing table in CLI |
| Objective | Verify that the router displays the routing table in the terminal. |
| Preconditions | Router R1 has already received and stored a routing table. |
| Input data | Command: show routing-table |
| Test steps | 1. Start router R1. 2. Receive routing table from controller. 3. Execute display command in router CLI. 4. Verify terminal output format. |
| Expected result | The routing table is displayed with columns: Destination, Prefix Length, Next Hop, Cost. All entries are shown. |
| Actual result | Router routing tables were displayed in the terminal with destination, prefix length, next hop and cost values. |
| Status | Passed |
| Evidence | Router CLI/terminal output and automated router-side tests. |
| Observations | The displayed table confirms that received routing information is usable by the router application. |

## TC-07: Update link cost and recalculate routes

| Field | Description |
| --- | --- |
| Test case ID | TC-07 |
| Related requirement | FR-08 |
| Test name | Update link cost and recalculate routes |
| Objective | Verify that the controller recalculates and redistributes affected routes after a link cost change. |
| Preconditions | Initial topology and routing tables already exist and were distributed. |
| Input data | Change cost R1-R3 from 5 to 1. |
| Test steps | 1. Load initial topology (R1-R2 cost 2, R2-R3 cost 1, R1-R3 cost 5). 2. Generate and distribute initial routing tables. 3. Record current routing table for R1. 4. Change link cost R1-R3 to 1 from administrator CLI. 5. Controller recalculates Dijkstra and redistributes affected tables. 6. Compare old and new routing table for R1. |
| Expected result | Routing table for R1 is updated: path to R3 is now R1 -> R3 (cost 1) instead of R1 -> R2 -> R3 (cost 3). |
| Actual result | The administrator executed link_cost_cli R1 R2 10. The controller updated the link cost, rebuilt the graph, recalculated shortest paths and stored 20 routes in the final five-router topology. |
| Status | Passed |
| Evidence | Terminal output from link_cost_cli showing LINK_COST_UPDATED and ROUTING_TABLES_STORED with saved_entries: 20. |
| Observations | This validates route recalculation after a weight change. |

## TC-08: Invalid JSON message handling

| Field | Description |
| --- | --- |
| Test case ID | TC-08 |
| Related requirement | NFR-05 |
| Test name | Invalid JSON message handling |
| Objective | Verify that the controller does not crash when it receives an invalid message. |
| Preconditions | Controller is running. |
| Input data | Malformed JSON message |
| Test steps | 1. Start controller. 2. Open a raw TCP connection to the controller. 3. Send malformed JSON string. 4. Observe controller behavior and logs. |
| Expected result | The controller rejects the message, logs an error with description, and continues running normally without crashing. |
| Actual result | Invalid and incomplete JSON messages were handled by returning ERROR responses or logging error events without crashing the controller. |
| Status | Passed |
| Evidence | Automated unittest results and eventos table entries for invalid AUTH, NEIGHBORS, HEARTBEAT and METRICS messages. |
| Observations | Validated through negative tests and error logging. |

TC-09: TLS encryption and token authentication

| Field | Description |
| --- | --- |
| Test case ID | TC-09 |
| Related requirement | FR-10, NFR-09 |
| Test name | TLS encryption and token authentication |
| Objective | Verify that all communication is encrypted with TLS and that routers without a valid token are rejected. |
| Preconditions | Controller is running with TLS configured. R1 has a valid token. A test client has an invalid token. |
| Input data | Valid token for R1. Invalid token: 'wrong_token_123'. |
| Test steps | 1. Start controller with TLS enabled. 2. Connect R1 with valid token. Verify AUTH_OK is received. 3. Connect a test client with invalid token. Verify AUTH_FAIL is received and connection is closed. 4. Verify successful TLS-protected connection using the configured SSL context and confirm that invalid-token clients are rejected. 5. Verify event is logged in controller database. |
| Expected result | R1 connects successfully. Invalid token client is rejected immediately. Message content is not readable without TLS decryption. |
| Actual result | The controller started as a TLS server and the router connected using TLS. Valid tokens produced AUTH_OK; invalid tokens were rejected with AUTH_FAIL. |
| Status | Passed |
| Evidence | Router output showing Connected to controller using TLS; automated token-validation tests. |
| Observations | TLS was demonstrated through the SSL-wrapped socket connection and certificate-based client configuration. |

TC-10: Heartbeat and fault detection

| Field | Description |
| --- | --- |
| Test case ID | TC-10 |
| Related requirement | FR-11, NFR-10 |
| Test name | Heartbeat and fault detection |
| Objective | Verify that the controller detects a router failure and automatically recalculates affected routes. |
| Preconditions | All 4-5 routers are running and registered. Routing tables have been distributed. |
| Input data | Router R4 is forcefully stopped (process killed). |
| Test steps | 1. Start all 5 routers and controller. 2. Verify routing tables are distributed correctly. 3. Forcefully stop router R4. 4. Wait for 3 missed heartbeat cycles according to the configured interval. 5. Verify controller marks R4 as 'caido' in database. 6. Verify controller recalculates Dijkstra excluding R4. 7. Verify affected routers receive updated routing tables. 8. Verify event is logged. |
| Expected result | After 3 missed heartbeats, R4 is marked as down. Routes that used R4 as next hop are recalculated and redistributed within acceptable time |
| Actual result | After a router process was stopped, router_health_cli marked the router as inactive and triggered route recalculation. |
| Status | Passed |
| Evidence | router_health_cli output showing inactive_routers_marked: 1 and route_recalculation_triggered: true; network_status_cli showing the router as inactivo. |
| Observations | Implementation uses router-originated HEARTBEAT messages and controller-side last-activity timeout. |

TC-11: LPM packet forwarding simulation

| Field | Description |
| --- | --- |
| Test case ID | TC-11 |
| Related requirement | FR-05, FR-06 |
| Test name | LPM packet forwarding simulation |
| Objective | Verify that each router applies Longest Prefix Matching correctly to determine the next hop for a simulated packet. |
| Preconditions | Router R1 has received and stored its routing table. Table contains at least two entries with overlapping prefixes. |
| Input data | Destination IP: 192.168.1.200. Forwarding table entries: 192.168.0.0/16 -> R2, 192.168.1.0/24 -> R3. |
| Test steps | 1. Load forwarding table into R1 with overlapping prefix entries. 2. Send PACKET_SIM message to R1 with destination 192.168.1.200. 3. Verify R1 selects the /24 entry (more specific) over the /16 entry. 4. Verify next hop is R3. 5. Trace full hop-by-hop path from R1 to destination. |
| Expected result | R1 selects next hop R3 based on /24 prefix (longest match). Full hop-by-hop path is displayed correctly. |
| Actual result | The router selected valid next hops from the received routing table during packet simulation. Known destinations were forwarded and unknown destinations were discarded. |
| Status | Passed |
| Evidence | packet_sim_cli output and metrics counters shown in network_status_cli. |
| Observations | Validated as forwarding behavior over locally stored routing tables. |

TC-12: FIFO queue and tail drop policy

| Field | Description |
| --- | --- |
| Test case ID | TC-12 |
| Related requirement | FR-12 |
| Test name | FIFO queue and tail drop policy |
| Objective | Verify that the router maintains a FIFO queue with a configurable maximum size and discards new packets when the queue is full. |
| Preconditions | Router R1 is running with queue max size configured to 5 packets. |
| Input data | 10 simulated packets sent in sequence to R1. |
| Test steps | 1. Configure R1 queue max size to 5. 2. Send 10 packets in rapid sequence to R1. 3. Verify that exactly 5 packets are queued. 4. Verify that 5 packets are discarded (tail drop). 5. Verify router queue state reflects 5 queued packets and 5 discarded packets, either in local persistence or runtime state.. |
| Expected result | First 5 packets are queued. Packets 6 to 10 are discarded. Database reflects correct states. |
| Actual result | The router maintained a FIFO queue with maximum size and tail drop behavior. When the queue limit was exceeded, packets were discarded and metrics were updated. |
| Status | Passed |
| Evidence | Automated router-side queue tests and network_status_cli metrics. |
| Observations | Queue size is configurable per router logic. |

TC-13: Metrics reporting to controller

| Field | Description |
| --- | --- |
| Test case ID | TC-13 |
| Related requirement | FR-13 |
| Test name | Metrics reporting to controller |
| Objective | Verify that each router sends periodic metrics to the controller and that the controller stores them correctly. |
| Preconditions | Controller and at least one router are running. Some packets have been processed. |
| Input data | Router R1 with queue size 3, 10 packets forwarded, 2 packets discarded. |
| Test steps | 1. Start controller and R1. 2. Process packets through R1 to generate metrics. 3. Wait for the configured metrics reporting cycle. 4. Verify R1 sends METRICS message: {"type": "METRICS", "router_id": "R1", "queue_size": 3, "forwarded": 10, "discarded": 2}. 5. Verify controller stores values in metricas_globales table with correct router_id and timestamp. |
| Expected result | Controller receives METRICS message from R1 and stores it correctly in metricas_globales. Values match the actual router state. |
| Actual result | Routers sent METRICS messages containing queue_size, packets_forwarded and packets_discarded. The controller stored these values in metricas_globales and displayed them in network_status_cli. |
| Status | Passed |
| Evidence | Router terminal output showing METRICS sent; controller response METRICS_OK; network_status_cli showing Queue, Forwarded and Discarded columns. |
| Observations | Validated through periodic reporting and final demo metrics. |

# 6. Requirements Traceability Matrix

| Requirement ID | Description | Test case ID | Test status |
| --- | --- | --- | --- |
| FR-01 | Router registration | TC-01 | Passed |
| FR-02 | Neighbor information exchange | TC-02 | Passed |
| FR-03 | Topology storage | TC-02 | Passed |
| FR-04 | Shortest path calculation (Dijkstra) | TC-03 | Passed |
| FR-05 | Routing table generation | TC-04, TC-11 | Passed |
| FR-06 | Routing table delivery and LPM forwarding | TC-05, TC-11 | Passed |
| FR-07 | Routing table display in CLI | TC-06 | Passed |
| FR-08 | Link cost update and route recalculation | TC-07 | Passed |
| FR-09 | Event logging in controller database | TC-09, TC-10 | Passed |
| FR-10 | TLS encryption of all communications | TC-09 | Passed |
| FR-11 | Heartbeat and automatic fault detection | TC-10 | Passed |
| FR-12 | FIFO queue with tail drop policy | TC-12 | Passed |
| FR-13 | Periodic metrics reporting to controller | TC-13 | Passed |
| NFR-05 | Invalid message handling | TC-08 | Passed |
| NFR-09 | TLS security — RSA handshake + AES session | TC-09 | Passed |
| NFR-10 | Availability — heartbeat fault detection | TC-10 | Passed |
| NFR-06 | Scalability with at least five routers | TC-14 | Passed |
| NFR-08 | Testability through automated and guided validation | TC-01 to TC-16 | Passed |

# 7. Test Execution Summary

| Total test cases | Passed | Failed | Pending | Success percentage |
| --- | --- | --- | --- | --- |
| 16 | 16 | 0 | 0 | 100% |

# 8. Final Test Report

The complete test process validated the final Sprint 3 system. All core requirements were executed through automated unittest cases, guided terminal execution, database verification and final five-router integration. No blocking issues remained before final delivery.

| Issue ID | Description | Severity | Related requirement | Suggested correction |
| --- | --- | --- | --- | --- |
| ISS-01 | No blocking issues remained after Sprint 3 validation. The final system passed the documented functional tests, guided TLS/security tests, five-router integration, path query, topology visualization and route recalculation demos. | None | All requirements | No corrective action required before final delivery. Keep the final GitHub repository and README as delivery evidence. |

# Sprint 1 Validation Note

For the final Sprint 3 delivery, the executed and validated tests cover Sprint 1, Sprint 2 and Sprint 3 functionality: registration, neighbor exchange, topology persistence, Dijkstra, routing table generation and delivery, LPM forwarding, link-cost update, TLS and token authentication, heartbeat fault detection, FIFO queue, metrics, five-router integration, path query and topology visualization.

## TC-14: Five-router integration test

| Field | Description |
| --- | --- |
| Test case ID | TC-14 |
| Related requirement | NFR-06, PB-14 |
| Test name | Five-router integration test |
| Objective | Verify that the controller manages five router applications simultaneously and computes complete routing information. |
| Preconditions | Controller and routers R1, R2, R3, R4 and R5 are configured and started. |
| Input data | Final topology: R1-R2 cost 10, R1-R3 cost 5, R2-R3 cost 1, R2-R4 cost 4, R3-R4 cost 2, R3-R5 cost 8, R4-R5 cost 3, R4-R1 cost 7, R5-R1 cost 6, R5-R2 cost 2. |
| Test steps | 1. Start controller. 2. Start routers R1-R5. 3. Execute network_status_cli. 4. Execute link_cost_cli R1 R2 10. 5. Verify saved_entries equals 20. |
| Expected result | All five routers appear as active and the controller stores 20 routes. |
| Actual result | R1, R2, R3, R4 and R5 were shown as active. link_cost_cli recalculated and stored 20 routes. |
| Status | Passed |
| Evidence | network_status_cli output and link_cost_cli output showing saved_entries: 20. |
| Observations | This test closes the final integration requirement with five routers. |

## TC-15: Query shortest path between selected routers

| Field | Description |
| --- | --- |
| Test case ID | TC-15 |
| Related requirement | FR-04, FR-05 |
| Test name | Query shortest path between selected routers |
| Objective | Verify that the administrator can request the current shortest path between two specific routers. |
| Preconditions | Five-router topology is active and routing information has been recalculated. |
| Input data | Commands: python -m app.cli.path_cli R1 R5 and python -m app.cli.path_cli R5 R4. |
| Test steps | 1. Execute path_cli R1 R5. 2. Verify path and cost. 3. Execute path_cli R5 R4. 4. Verify path and cost. |
| Expected result | The system prints the selected source, destination, total cost and path. |
| Actual result | R1 to R5 returned R1 -> R3 -> R4 -> R5 with cost 10. R5 to R4 returned R5 -> R2 -> R3 -> R4 with cost 5. |
| Status | Passed |
| Evidence | Terminal output from path_cli. |
| Observations | This command supports live professor-selected path queries during the demo. |

## TC-16: Visual topology diagram generation

| Field | Description |
| --- | --- |
| Test case ID | TC-16 |
| Related requirement | NFR-04, NFR-06 |
| Test name | Visual topology diagram generation |
| Objective | Verify that the administrator can generate a visual drawing of the active directed weighted topology. |
| Preconditions | Active topology exists in the controller database. |
| Input data | Command: python -m app.cli.topology_diagram_cli. |
| Test steps | 1. Execute topology_diagram_cli. 2. Verify terminal topology summary. 3. Open outputs/network_topology.svg. |
| Expected result | The system generates an SVG diagram showing routers, directed links and costs. |
| Actual result | outputs/network_topology.svg was generated and opened successfully, showing R1-R5 with directed links and cost labels. |
| Status | Passed |
| Evidence | Generated SVG topology diagram and terminal output. |
| Observations | This test supports the required visual topology demonstration. |

# Final Validation Notes

Controller tests were executed with Python unittest and reached a successful final result with no failing test cases.

The final integration demo used five routers: R1, R2, R3, R4 and R5.

Route recalculation after link-cost update produced 20 saved routes, matching five routers with four destinations each.

The health-check flow marked a stopped router as inactive and triggered route recalculation.

The topology diagram command generated an SVG drawing of the active directed weighted graph.

The final project was pushed to the GitHub repository after validation.