System Architecture and Requirements

Centralized Routing Management System

| Course | Informatics for Telecommunications |
| --- | --- |
| Program | Electronic Engineering and Telecommunications |
| Project | Centralized Routing System |
| Team name | NetCore Team |
| Team members | Isabella Perez Hoyos (Documenter + Developer + Tester) |
| Professor | Oscar Mauricio Caicedo Rendon |
| Date | April 25, 2026 |
| Version | 1.0 |

# 1. Introduction

## 1.1 Purpose of the Document

This document describes the requirements, architecture, components, communication mechanisms, data structures, and testing strategy of the centralized routing management system developed as part of the Informatics for Telecommunications course. It serves as the primary technical reference for development, integration, and evaluation of the system.

## 1.2 Project Context

The project consists of developing a software-based centralized routing system in Python. The system includes a controller application (Control Plane) and several router applications (Data Plane). The controller collects topology information from routers, computes optimal routing paths using Dijkstra's algorithm, generates forwarding tables, and distributes them securely to each router via TCP with TLS encryption and JSON messages. Each router applies Longest Prefix Matching to forward simulated packets.

## 1.3 Project Scope

| Included | Not included |
| --- | --- |
| Controller application | Real physical routers |
| Router applications | Real packet forwarding |
| TCP communication | Internet-scale routing |
| JSON-based messages | Full BGP, OSPF, or RIP implementation |
| Shortest path calculation (Dijkstra) | Hardware-level routing |
| Forwarding table generation and distribution | Production-grade SDN controller |
| Longest Prefix Matching (LPM) forwarding | Graphical user interface |
| Token-based authentication | High availability / redundant controller |
| Heartbeat and fault detection | Support for more than 10 simultaneous routers |
| FIFO queue and metrics reporting | OpenFlow protocol implementation |

# 2. Objectives

General objective: Develop a centralized routing system in Python that allows router applications to register with a controller, send topology information, receive forwarding tables, and simulate routing decisions based on shortest path computation using Dijkstra's algorithm, with secure TCP+TLS communication.

Specific Objectives

- Design the architecture of a centralized routing system composed of a controller application and multiple router applications following the MVC+DAO pattern.

- Implement secure communication between routers and the controller using TCP sockets with TLS encryption and JSON messages.

- Implement Dijkstra's algorithm to compute shortest paths and generate forwarding tables with Longest Prefix Matching support.

- Implement token-based authentication and heartbeat fault detection mechanisms.

- Design and implement a distributed database: one centralized persistent database for the controller and one local database per router.

# 3. System Overview

The centralized routing system follows a controller-based distributed architecture, similar to the basic principles of Software-Defined Networking (SDN) at an educational level. Router applications do not compute complete network routes independently. Each router sends its local topology information to the controller. The controller builds a global view of the network, computes optimal routes, and distributes forwarding tables back to each router.

| Component | Description |
| --- | --- |
| Controller application | Central entity that manages topology, computes routes, and sends routing tables |
| Router application | Software entity that registers with the controller and receives routing information, applies LPM for packet forwarding, and reports metrics |
| Communication module | Handles TCP+TLS message exchange between controller and routers using JSON schemas |
| Topology manager | Stores and updates the network topology as a weighted graph. Used as input for Dijkstra |
| Routing algorithm module | Computes shortest paths from each router to all others using Dijkstra's algorithm |
| Routing table manager | Generates forwarding table entries (destination prefix, prefix length, next hop, cost) from Dijkstra results |
| CLI module | Allows the administrator to interact with the controller and view router state from the command line |
| Logging module | Records relevant system events: registrations, topology updates, route computations, faults, and metrics |
| Queue manager | Manages FIFO packet queue with configurable max size and tail drop policy in each router |
| Heartbeat module | Monitors router connectivity. Marks a router as down after 3 consecutive missed responses and triggers route recalculation |

# 4. Functional Requirements

| ID | Requirement | Description | Priority |
| --- | --- | --- | --- |
| FR-01 | Router registration | Routers must register with the controller by sending router ID, IP address, port, and authentication token. | High |
| FR-02 | Neighbor information exchange | Routers must send neighbor and link cost information to the controller to build the network topology. | High |
| FR-03 | Topology storage | The controller must store the network topology as a weighted directed graph (adjacency dictionary). | High |
| FR-04 | Route computation | The controller must compute shortest paths using Dijkstra's algorithm from each router to all other routers. | High |
| FR-05 | Routing table generation | The controller must generate a forwarding table for each router containing: destination prefix, prefix length, next hop, and cost. | High |
| FR-06 | Routing table delivery | The controller must send the corresponding forwarding table to each router. Routers store it in their local database. | High |
| FR-07 | Routing table visualization | Routers must display their forwarding tables through the command-line interface showing destination, prefix length, next hop, and cost. | Medium |
| FR-08 | Link cost update | The system must allow link cost updates from the administrator CLI. The controller recalculates and redistributes affected routing tables automatically. | Medium |
| FR-09 | Event logging | The system must log registrations, topology updates, route computations, table deliveries, and fault events in the controller database. | Low |
| FR-10 | TLS encryption | All communication between the controller and routers must be encrypted with TLS. RSA for handshake, AES for session messages. | High |
| FR-11 | Heartbeat fault detection | The controller sends a heartbeat every 5 seconds. After 3 consecutive missed responses, the router is marked as down and affected routes are recalculated. | High |
| FR-12 | FIFO queue management | Each router maintains a FIFO packet queue with configurable maximum size. When full, new packets are discarded (tail drop). | Medium |
| FR-13 | Metrics reporting | Each router sends periodic metrics (queue size, packets forwarded, packets discarded) to the controller every 30 seconds. | Medium |

# 5. Non-Functional Requirements

| ID | Requirement | Description | Priority |
| --- | --- | --- | --- |
| NFR-01 | Python implementation | The system must be implemented in Python. | High |
| NFR-02 | Modular design | Code organized into modules following MVC+DAO pattern: controllers/, models/, views/, dao/, services/, network/. | High |
| NFR-03 | JSON messages | All messages between components must use valid JSON objects with a defined type field. | High |
| NFR-04 | Command-line interface | Controller and routers must be started and operated from the terminal with clear command-line arguments. | High |
| NFR-05 | Error handling | Invalid, malformed, or unknown messages must be handled gracefully without crashing the system. | Medium |
| NFR-06 | Scalability | The system must support at least 5 router applications simultaneously. Target demo topology: 5 routers. | Medium |
| NFR-07 | Maintainability | All functions and classes must include docstrings and meaningful names. Code must be readable and commented | Medium |
| NFR-08 | Testability | All main requirements must have documented test cases. Tests automated with Python unittest after first guided execution. | Medium |
| NFR-09 | Security — TLS | All TCP connections use TLS. RSA for handshake authentication. AES for session encryption. Connections without valid token are rejected. | High |
| NFR-10 | Availability | Heartbeat every 5 seconds. Three consecutive failures mark a router as down and trigger automatic route recalculation. | Medium |

# 6. System Architecture

## 6.1 Architectural Style

The system follows a distributed client-server architecture with two levels, inspired by the principles of Software-Defined Networking (SDN). The Control Plane (controller) acts as the server and the Data Plane (routers) act as clients. This architecture was chosen over alternatives because:

- Client-server simple: does not separate routing logic from data access, making it hard to maintain.

- Peer-to-peer: eliminates centralization, which is the core objective of the system.

- Microservices: over-engineered for the current project scope and timeline.

- Distributed client-server: reflects the real physical separation between a central controller and distributed routers. Internally, each component uses the MVC+DAO layered pattern.

## 6.2 Architecture Diagram

6.3 Internal structure: APP CONTROLLER

| Layer | Components | Responsibility |
| --- | --- | --- |
| View | controller_cli_view.py, topology_view.py, routing_table_view.py, log_view.py | Display topology, routing tables, and metrics. Receive administrator commands. |
| Controller | router_registry_controller.py, topology_controller.py, routing_controller.py, communication_controller.py | Handle registration, build topology, execute Dijkstra, distribute tables, manage heartbeat. |
| Model | router.py, link.py, topology.py, routing_table.py, routing_table_entry.py | Represent system entities without business logic. |
| DAO | router_dao.py, topology_dao.py, routing_table_dao.py, log_dao.py | Only layer that accesses the controller database. CRUD operations. |
| Services | dijkstra_service.py, routing_table_service.py, message_service.py | Encapsulate complex business logic: Dijkstra execution, table generation, JSON parsing. |
| Network | tcp_server.py, client_handler.py | Manage TCP+TLS server socket and per-client connection threads. |

6.4 Internal structure: APP ROUTER

| Layer | Components | Responsibility |
| --- | --- | --- |
| View | router_cli_view.py, routing_table_view.py, forwarding_view.py | Display local routing table, queue state, and metrics. |
| Controller | registration_controller.py, routing_table_controller.py, forwarding_controller.py, queue_controller.py | Handle registration, receive and store table, apply LPM, manage FIFO queue. |
| Model | router.py, routing_table_entry.py, packet.py, metric.py | Represent local router entities. |
| DAO | routing_table_dao.py, queue_dao.py, metric_dao.py | Access local SQLite database: forwarding_table, cola_paquetes, metricas. |
| Services | forwarding_service.py, message_service.py, metric_service.py | LPM logic, JSON parsing, metrics capture and reporting. |
| Network | tcp_client.py, controller_connection.py | Manage TCP+TLS client socket connecting to controller. |

# 7. Component Description

| Component | Main responsibility | Input | Output |
| --- | --- | --- | --- |
| Controller application | Manage global network information and compute routes | Router registration, topology updates, link cost changes. | Forwarding tables, AUTH_OK/FAIL, HEARTBEAT, events. |
| Router application | Represent a simulated router: register, receive table, forward packets, report metrics. | Forwarding table from controller, PACKET_SIM, HEARTBEAT. | AUTH message, NEIGHBORS, METRICS, ACK. |
| Topology manager | Store and update the network topology as a weighted graph. | Router nodes, links with costs. | Weighted adjacency dictionary for Dijkstra. |
| Dijkstra service | Compute shortest paths from each router to all others. | Weighted graph (adjacency dictionary). | Dictionary of shortest paths and costs per router. |
| Queue manager | Maintain FIFO packet queue with tail drop policy in each router. | Incoming simulated packets. | Queued or discarded status, queue size metrics. |
| Routing table manager | Convert Dijkstra paths into forwarding table entries. | Shortest paths and costs. | Forwarding table: destination, prefix length, next hop, cost. |
| Heartbeat module | Detect router failures and trigger route recalculation. | Periodic HEARTBEAT responses from routers. | Router status update, ROUTER_DOWN event, route recalculation. |

# 8. Communication Design

| Field | Description |
| --- | --- |
| Protocol | TCP — chosen over UDP because configuration messages cannot be lost or arrive out of order. |
| Security layer | TLS over TCP. |
| Data format | JSON — lightweight, readable, and compatible with any language. |
| Encoding | UTF-8 |
| Communication model | Client-server. Controller is the server (port 6000). Each router is a client connecting on startup. |
| Concurrency | Each router connection is handled in an independent thread on the controller side (threading module). |
| Authentication | Each router sends a unique token on connection. Controller validates before accepting. Invalid tokens result in AUTH_FAIL and connection close. |

| Message type | Sender | Receiver | Purpose |
| --- | --- | --- | --- |
| AUTH | Router | Controller | Register router and authenticate with token. |
| AUTH_OK | Controller | Router | Confirm successful authentication. |
| AUTH_FAIL | Controller | Router | Reject invalid token. Connection is closed. |
| NEIGHBORS | Router | Controller | Send neighbor list and link costs to build topology. |
| UPDATE_TABLE | Controller | Router | Send complete forwarding table to router. |
| HEARTBEAT | Router | Controller | Periodic availability report. The controller updates last activity for fault detection. |
| HEARTBEAT_OK | Controller | Router | Response confirming that the controller received the heartbeat. |
| METRICS | Router | Controller | Periodic report: queue size, packets forwarded, packets discarded. |
| PACKET_SIM | Controller | Router | Trigger simulated packet forwarding to a destination IP. |
| ROUTER_DOWN | Controller | All routers | Notify that a router has been marked as down and routes have been recalculated. |
| LINK_COST_UPDATE | Admin CLI | Controller | Update the cost of a specific link and trigger recalculation. |
| ERROR | Controller / Router | Controller / Router | Report an invalid or unrecognized message. |

# 9. Example JSON Messages

Router authentication (AUTH):

{

"type": "AUTH",

"router_id": "R1",

"ip": "127.0.0.1",

"port": 5001,

"token": "a3f8c2d1e9b7..."

}

Topology update (NEIGHBORS):

{

"type": "NEIGHBORS",

"router_id": "R1",

"neighbors": [

{"neighbor_id": "R2", "cost": 10},

{"neighbor_id": "R3", "cost": 5}

]

}

Forwarding table delivery (UPDATE_TABLE):

{

"type": "UPDATE_TABLE",

"router_id": "R1",

"entries": [

{"prefix": "192.168.2.0", "prefix_len": 24, "next_hop": "R2", "cost": 10},

{"prefix": "192.168.3.0", "prefix_len": 24, "next_hop": "R3", "cost": 5}

]

}

Metrics report (METRICS):

{

"type": "METRICS",

"router_id": "R1",

"queue_size": 3,

"packets_forwarded": 42,

"packets_discarded": 2

}

Link cost update (LINK_COST_UPDATE):

{

"type": "LINK_COST_UPDATE",

"source": "R1",

"destination": "R3",

"new_cost": 1

}

# 10. Data Model

10.1 Controller Database (centralized and persistent)

| Entity / Table | Field | Type | Description |
| --- | --- | --- | --- |
| routers | id, nombre, ip, puerto, token, estado, ultima_actividad | INT, VARCHAR, VARCHAR, INT, VARCHAR, VARCHAR, DATETIME | Registered routers. Estado: 'activo' or 'inactivo'. ultima_actividad stores the latest heartbeat timestamp. |
| enlaces | id, router_origen_id, router_destino_id, costo, estado | INT, INT (FK), INT (FK), INT, ENUM | Network links. Weighted directed edges for Dijkstra. |
| rutas_calculadas | id, router_origen_id, router_destino_id, costo_total, calculado_en | INT, INT (FK), INT (FK), INT, DATETIME | Routes computed by Dijkstra. The controller stores origin, destination, total cost and calculation timestamp after each recalculation. |
| metricas_globales | id, router_id, router_nombre, timestamp, tamano_cola, paquetes_enviados, paquetes_descartados | INT, INT (FK), VARCHAR, DATETIME, INT, INT, INT | Periodic metric reports received from routers and displayed by network_status_cli. |
| eventos | id, tipo, descripcion, timestamp | INT, ENUM, VARCHAR, DATETIME | System event log. No FK because some events are system-wide. |

10.2 Router Local Database (one per router, dynamic)

| Table | Fields | Types | Description |
| --- | --- | --- | --- |
| forwarding_table | id, prefijo_destino, longitud_prefijo, siguiente_salto, costo, actualizado_en | INT, VARCHAR, INT, VARCHAR, INT, DATETIME | Received from controller and stored locally by the router for forwarding simulation. prefix_length enables LPM behavior. |
| cola_paquetes | id, ip_origen, ip_destino, timestamp_llegada, estado, siguiente_salto | INT, VARCHAR, VARCHAR, DATETIME, ENUM, VARCHAR | Local queue state and packet processing history. Estado values include queued, sent/forwarded, and discarded depending on the simulation result. |
| metricas | id, timestamp, tamano_cola, paquetes_enviados, paquetes_descartados | INT, DATETIME, INT, INT, INT | Periodic snapshots of queue size, packets forwarded and packets discarded. Summaries are sent to controller as METRICS messages. |

# 11. Suggested Projects Structure

11.1 Controller

centralized-routing-controller/

|-- app/

|   |-- main.py

|   |-- controllers/

|   |   |-- router_registry_controller.py

|   |   |-- topology_controller.py

|   |   |-- routing_controller.py

|   |   |-- heartbeat_controller.py

|   |   `-- communication_controller.py

|   |-- models/

|   |   |-- router.py

|   |   |-- link.py

|   |   |-- topology.py

|   |   |-- routing_table.py

|   |   `-- routing_table_entry.py

|   |-- views/

|   |   |-- controller_cli_view.py

|   |   |-- topology_view.py

|   |   |-- routing_table_view.py

|   |   `-- log_view.py

|   |-- dao/

|   |   |-- router_dao.py

|   |   |-- topology_dao.py

|   |   |-- routing_table_dao.py

|   |   |-- metrics_dao.py

|   |   `-- log_dao.py

|   |-- services/

|   |   |-- dijkstra_service.py

|   |   |-- routing_table_service.py

|   |   `-- message_service.py

|   |-- network/

|   |   |-- tcp_server.py

|   |   `-- client_handler.py

|   `-- utils/

|       |-- constants.py

|       `-- logger.py

|-- tests/

|   |-- test_router_registry.py

|   |-- test_dijkstra_service.py

|   |-- test_routing_table_service.py

|   |-- test_topology.py

|   `-- test_controller_communication.py

|-- docs/

|-- README.md

`-- requirements.txt

11.2 Router

centralized-routing-router/

|-- app/

|   |-- main.py

|   |-- controllers/

|   |   |-- registration_controller.py

|   |   |-- routing_table_controller.py

|   |   |-- forwarding_controller.py

|   |   |-- queue_controller.py

|   |   `-- metric_controller.py

|   |-- models/

|   |   |-- routing_table_entry.py

|   |   |-- packet.py

|   |   `-- metric.py

|   |-- views/

|   |   |-- router_cli_view.py

|   |   |-- routing_table_view.py

|   |   `-- forwarding_view.py

|   |-- dao/

|   |   |-- routing_table_dao.py

|   |   |-- queue_dao.py

|   |   `-- metric_dao.py

|   |-- services/

|   |   |-- forwarding_service.py

|   |   |-- message_service.py

|   |   `-- metric_service.py

|   |-- network/

|   |   |-- tcp_client.py

|   |   `-- controller_connection.py

|   `-- utils/

|       |-- constants.py

|       `-- logger.py

|-- tests/

|   |-- test_forwarding_service.py

|   |-- test_queue_manager.py

|   `-- test_router_communication.py

|-- docs/

|-- README.md

`-- requirements.txt

# 12. Routing Algorithm Description

12.1 Dijkstra's Algorithm

The system uses Dijkstra's algorithm to compute the shortest path from each router to all other routers in the network. The controller applies the algorithm over the global network topology stored as a weighted directed graph.

The algorithm works as follows: starting from a source router, it assigns a tentative cost of zero to the source and infinity to all others. It then visits the unvisited router with the lowest tentative cost, updates the costs of its neighbors if a shorter path is found through the current router, and repeats until all routers are visited.

After computing shortest paths from a source router, the controller extracts the next hop for each destination and builds the forwarding table for that router. This process is repeated for every router in the network.

12.2 Example

Topology: R1-R2 cost 2, R2-R3 cost 1, R1-R3 cost 5, R1-R4 cost 3, R4-R3 cost 1.

| Source | Destination | Total cost | Path |
| --- | --- | --- | --- |
| R1 | R2 | 2 | R1 → R2 |
| R1 | R3 | 3 | R1 → R2 → R3 |
| R1 | R4 | 3 | R1 → R4 |

Resulting forwarding table for R1: to reach R2, next hop R2. To reach R3, next hop R2 (cost 3 via R2 < cost 5 direct). To reach R4, next hop R4.

12.3 Longest Prefix Matching (LPM)

Each router applies LPM when a simulated packet arrives. The forwarding table is queried ordered by prefix length descending. The first entry whose prefix matches the destination IP is selected. The prefix_length field is stored as a separate integer in the database specifically to enable efficient ORDER BY prefix_length DESC queries without string processing.

# 13. Sequence Diagram

Router R1                     Controller

|                               |

|--- AUTH (id, ip, port, token)->|

|<-------- AUTH_OK -------------|

|                               |

|--- NEIGHBORS (links+costs) --->|

|                               | Build topology graph

|                               | Run Dijkstra

|                               | Generate forwarding tables

|                               |

|<------ UPDATE_TABLE ----------|

| Store in local DB             |

|                               |

|<------ HEARTBEAT -------------|  (every 5 seconds)

|------- HEARTBEAT_ACK -------->|

|                               |

|------- METRICS -------------->|  (every 30 seconds)

|                               | Store in metricas_globales

|                               |

| (R4 stops responding)         |

|                               | 3 missed heartbeats

|                               | Mark R4 as 'caido'

|                               | Re-run Dijkstra

|<------ UPDATE_TABLE ----------|  (updated routes)

|<------ ROUTER_DOWN -----------|

# 14. Test Strategy

| Test area | Description | Automation |
| --- | --- | --- |
| Router registration | Verify that routers register correctly and are stored in the database. | unittest — verify database entry after registration. |
| Topology update | Verify that topology information is received and stored as a weighted graph. | unittest — verify graph adjacency after NEIGHBORS message. |
| Dijkstra algorithm | Verify that the routing algorithm returns correct shortest paths. | Pure unit test — no network required. Fastest to automate. |
| Routing table generation | Verify destination, next hop, prefix length, and cost entries. | unittest — compare generated table against expected output. |
| Routing table delivery | Verify that each router receives and stores its forwarding table. | unittest — verify local database after UPDATE_TABLE message. |
| LPM forwarding | Verify that the router selects the most specific matching prefix. | Pure unit test of forwarding_service.py. |
| TLS and authentication | Verify encryption is active and invalid tokens are rejected. | Automated for token validation. TLS verified manually on first run. |
| Heartbeat and fault detection | Verify router is marked down after 3 missed heartbeats and routes recalculate. | unittest with simulated timeout. Timing-sensitive. |
| FIFO queue and tail drop | Verify queue limit and packet discard behavior. | Pure unit test of queue_controller.py. |
| Error handling | Verify invalid messages do not crash the system. | unittest — send malformed JSON and verify system continues. |
| Link cost update | Verify that an administrator changes a link cost and the controller recalculates routes. | Guided CLI test with link_cost_cli and unittest coverage. |
| Metrics reporting | Verify queue size, packets forwarded and packets discarded are stored and displayed. | Guided execution and unittest coverage for metrics controller/DAO. |
| Five-router integration | Verify R1-R5 are active and routing calculation stores 20 routes. | Guided final demo with network_status_cli and link_cost_cli. |
| Path query | Verify the system can display the current shortest path between two selected routers. | Guided CLI test with path_cli. |
| Topology visualization | Verify the active topology can be generated as a directed weighted diagram. | Guided CLI test with topology_diagram_cli generating SVG output. |

# 15. Conclusions

This section is completed after final system delivery (Sprint 3 — June 17, 2025).

- Was the system implemented successfully? Did all functional requirements reach their acceptance criteria?

- Did the controller correctly compute routing tables using Dijkstra's algorithm for all routers in the test topology?

- Did the routers receive, store, and correctly apply Longest Prefix Matching for simulated packet forwarding?

- Was the TLS encryption and token authentication successfully implemented and validated?

- Did the heartbeat mechanism correctly detect router failures and trigger automatic route recalculation?

- What were the main technical challenges encountered during development?

- What improvements or additional features could be added in future versions of the system?

# Sprint 1 Implementation Update

Implemented scope in Sprint 1. The current implementation delivers the communication base of the centralized routing system: controller start-up, router start-up, TCP communication, JSON message exchange, router registration, neighbor exchange, topology storage, and event logging in the controller database.

## Current Execution and Configuration Model

• The controller uses an external JSON configuration file: config/controller_config.json.

• Each router instance uses its own JSON configuration file, for example: config/router_r1.json, config/router_r2.json, config/router_r3.json, config/router_r4.json, and config/router_r5.json.

• This configuration model allows the system to run on different computers without modifying the Python source code; only configuration files need to be changed.

• For local validation, the system can run with 127.0.0.1. For distributed execution, only the controller_host and router-related addresses in the JSON configuration files must be updated.

## Messages Implemented in Sprint 1

• AUTH

• AUTH_OK

• AUTH_FAIL

• NEIGHBORS

• NEIGHBORS_OK

• ERROR

## Controller Persistence Implemented in Sprint 1

• routers table: stores registered routers and their connection data.

• enlaces table: stores the weighted topology edges received from routers.

• eventos table: stores registration and topology-update events.

## Validated Sprint 1 Flow

• R2 registers with the controller.

• R3 registers with the controller.

• R1 registers with the controller.

• R1 sends neighbors R2 (cost 10) and R3 (cost 5).

• The controller stores the links in enlaces and records the corresponding events in eventos.

## Clarification About Planned Features

The original architecture roadmap is now implemented for the final Sprint 3 delivery. TLS, Dijkstra, routing table generation, routing table delivery, heartbeat, queueing, metrics, link-cost update, path query, topology diagram, and five-router integration were completed and validated.

# Sprint 2 Implementation Update

Sprint 2 completed the routing and forwarding capabilities of the system. The controller builds the active topology graph from the database, executes Dijkstra, generates routing tables per router and sends UPDATE_TABLE messages to router applications. Routers store the received forwarding tables locally and use them to simulate forwarding decisions.

## Implemented Sprint 2 Components

routing_controller.py coordinates graph loading, shortest path calculation, table generation and route persistence.

dijkstra_service.py computes shortest paths from each router to all reachable destinations.

routing_table_service.py converts shortest paths into routing table entries with destination, prefix length, next hop and cost.

routing_table_dao.py stores calculated route information in the controller database.

router-side routing_table_controller.py and routing_table_dao.py store received UPDATE_TABLE data locally.

forwarding logic applies the stored table to simulate next-hop decisions and packet drops when no route exists.

# Sprint 3 Implementation Update

Sprint 3 completed the security, monitoring, fault tolerance, queue management, metrics, visualization and final integration capabilities required for the final delivery.

## Implemented Sprint 3 Components

tcp_server.py and tcp_client.py use Python ssl to wrap TCP connections with TLS.

generate_certificates.py creates project test certificates used by the controller and router clients.

router_registry_controller.py validates AUTH messages with router token values and rejects invalid tokens.

controller_connection.py sends HEARTBEAT and METRICS messages periodically from each router.

router_health_cli.py marks inactive routers after missing heartbeat activity and triggers route recalculation.

link_cost_cli.py updates link weights and triggers route recalculation.

queue_controller.py and queue_dao.py implement FIFO queue behavior and tail drop.

metric_controller.py, metric_dao.py and metric_service.py collect, store and report queue size, packets forwarded and packets discarded.

network_status_cli.py displays router state, heartbeat activity and metrics in one administrator view.

path_cli.py displays the shortest path between two selected routers.

topology_diagram_cli.py generates an SVG diagram of the active directed weighted topology.

## Final Five-Router Demo Topology

R1 -> R2 cost 10 and R1 -> R3 cost 5.

R2 -> R3 cost 1 and R2 -> R4 cost 4.

R3 -> R4 cost 2 and R3 -> R5 cost 8.

R4 -> R5 cost 3 and R4 -> R1 cost 7.

R5 -> R1 cost 6 and R5 -> R2 cost 2.

## Final Validation Evidence

network_status_cli displayed R1, R2, R3, R4 and R5 as active in the final demo.

link_cost_cli R1 R2 10 recalculated routes and stored 20 entries in the final topology.

path_cli R1 R5 returned R1 -> R3 -> R4 -> R5 with total cost 10.

path_cli R5 R4 returned R5 -> R2 -> R3 -> R4 with total cost 5.

topology_diagram_cli generated outputs/network_topology.svg showing routers, directed edges and link costs.

router_health_cli marked a stopped router as inactive and triggered route recalculation.

The final project was committed and pushed to GitHub.

# Final Architecture Status

The final architecture satisfies the documented control-plane/data-plane design. The controller manages global topology and routing computation, while routers register, receive tables, simulate forwarding, report metrics and send heartbeat messages. Communication uses TCP, TLS and JSON. The project remains within the documented educational scope: simulated routers, simulated forwarding, command-line operation and no production-grade SDN controller or real hardware routing.