Requirements Test Plan

Centralized Routing Management System

| Course | Informatics for Telecommunications |
| --- | --- |
| Program | Electronic Engineering and Telecommunications |
| Project | Centralized Routing System |
| Team name | NetCore Team |
| Team members | Eider Daniel Fernandez (Developer + Tester), Isabella Perez Hoyos (Documenter + Developer) |
| Professor | Oscar Mauricio Caicedo Rendon |
| Date | April 25, 2026 |
| Version | 1.0 |

# 1. Test Plan Objective

The objective of this test plan is to verify that the centralized routing system satisfies all defined functional and non-functional requirements. The tests validate router registration, topology collection, routing algorithm execution (Dijkstra), routing table generation and distribution, Longest Prefix Matching forwarding, secure communication via TCP with TLS encryption, fault detection through heartbeat, queue management, and metrics reporting. Each test case was initially executed manually and guided. After the first successful execution, tests are automated using Python's unittest framework to ensure continuous validation throughout the development process.

# 2. Testing Scope

| Included in testing | Status |
| --- | --- |
| Controller application | Yes |
| Router application | Yes |
| Communication protocol | Yes |
| JSON message format | Yes |
| Topology storage | Yes |
| Routing algorithm (Dijkstra) | Yes |
| Routing table generation and distribution | Yes |
| Command-line interface | Yes |
| Error handling | Yes |
| Longest Prefix Matching (LPM) forwarding | Yes |
| TLS encryption and token authentication | Yes |
| Heartbeat and fault detection | Yes |
| FIFO queue and tail drop policy | Yes |
| Metrics reporting and storage | Yes |
| Test automation (Python unittest) | Yes |

Priority will be given to automated testing of core routing, forwarding, topology, and messaging behaviors. Security and monitoring tests may combine guided execution and partial automation.

# 3. Test Environment

| Element | Description |
| --- | --- |
| Operating system | Windows 11 |
| Python version | Python 3.12 |
| Communication protocol | socket, json, threading, pytest, mysql-connector-python |
| Libraries used | socket, ssl, json, threading, unittest |
| Number of router applications | 4-5 (R1, R2, R3, R4 - R5) — partial mesh topology |
| Controller IP address | Localhost for local testing |
| Controller port | 6000 |

# 4. Test Case Template

| Field | Description |
| --- | --- |
| Test case ID | TC-XX |
| Related requirement | FR-XX or NFR-XX |
| Test name | Short name of the test |
| Objective | What the test verifies |
| Preconditions | Conditions required before executing the test |
| Input data | Data used during the test |
| Test steps | Ordered steps to execute the test |
| Expected result | Correct system behavior |
| Actual result | Observed behavior |
| Status | Passed / Failed |
| Evidence | Screenshot, log, file, or terminal output |
| Observations | Additional comments |

# 5. Functional Test Cases

## TC-01: Router registration with the controller

| Field | Description |
| --- | --- |
| Test case ID | TC-01 |
| Related requirement | FR-01 |
| Test name | Router registration with the controller |
| Objective | Verify that a router can register successfully with the controller. |
| Preconditions | Controller application is running. |
| Input data | Router ID: R1, IP: 127.0.0.1, Port: 5001 |
| Test steps | 1. Start the controller. 2. Start router R1. 3. Send registration message: {"type": "AUTH", "id": "R1", "ip": "127.0.0.1", "port": 5001, "token": "<token_R1>"}. 4. Check controller registry in database table routers. |
| Expected result | The controller stores R1 successfully and confirms registration with AUTH_OK message. |
| Actual result | Controller received AUTH from router R1, responded with AUTH_OK, and stored the router successfully in the routers table. |
| Status | Passed |
| Evidence | Terminal output from controller and router. SQL query to routers and eventos tables confirming registration. |
| Observations | Validated through integrated execution with the running controller and router instances. This is part of the current Sprint 1 working code |

## TC-02: Send neighbor information to controller

| Field | Description |
| --- | --- |
| Test case ID | TC-02 |
| Related requirement | FR-02, FR-03 |
| Test name | Send neighbor information to controller |
| Objective | Verify that routers can send neighbor and link cost information and the controller builds the topology graph. |
| Preconditions | Controller and routers R1, R2, R3 are running and registered. |
| Input data | R1 neighbors: R2 cost 10, R3 cost 5 |
| Test steps | 1. Start controller. 2. Start routers R1, R2, and R3. 3. Send neighbor information from R1: {"type": "NEIGHBORS", "from": "R1", "links": [{"to": "R2", "cost": 10}, {"to": "R3", "cost": 5}]}. 4. Check topology graph stored in controller (table enlaces). |
| Expected result | The controller stores links R1-R2 (cost 10) and R1-R3 (cost 5) in the topology graph. |
| Actual result | The controller received NEIGHBORS from R1 and stored links R1-R2 (cost 10) and R1-R3 (cost 5) in the enlaces table. |
| Status | Passed |
| Evidence | Terminal output from controller and router. SQL query to enlaces and eventos tables confirming topology storage. |
| Observations | Validated with three configured router instances (R1, R2, R3). |

## TC-03: Dijkstra shortest path calculation

| Field | Description |
| --- | --- |
| Test case ID | TC-03 |
| Related requirement | FR-04 |
| Test name | Dijkstra shortest path calculation |
| Objective | Verify that the controller computes the shortest path correctly using Dijkstra's algorithm. |
| Preconditions | Network topology is stored in the controller. |
| Input data | Topology: R1-R2 cost 2, R2-R3 cost 1, R1-R3 cost 5 |
| Test steps | 1. Load topology into controller graph. 2. Execute Dijkstra from R1. 3. Check shortest path from R1 to R3. 4. Verify cost and path sequence. |
| Expected result | The shortest path from R1 to R3 is R1 -> R2 -> R3 with total cost 3. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | This test can be executed as a pure unit test of the Dijkstra function without network connections. Automated with unittest. |

## TC-04: Generate routing table for each router

| Field | Description |
| --- | --- |
| Test case ID | TC-04 |
| Related requirement | FR-05 |
| Test name | Generate routing table for each router |
| Objective | Verify that the controller generates correct routing tables from Dijkstra results. |
| Preconditions | Topology is available and shortest paths were computed. |
| Input data | Network topology with at least four routers (R1, R2, R3, R4). |
| Test steps | 1. Load topology with 4 routers. 2. Execute Dijkstra algorithm. 3. Generate routing table for R1. 4. Verify each entry contains: destination prefix, prefix length, next hop, and cost. |
| Expected result | Routing table for R1 contains valid entries for all reachable destinations with correct next hop and cost. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Automated after first execution. Verifies table entries against expected Dijkstra output. |

## TC-05: Send routing table to router

| Field | Description |
| --- | --- |
| Test case ID | TC-05 |
| Related requirement | FR-06 |
| Test name | Send routing table to router |
| Objective | Verify that the controller sends the correct routing table to each router via TCP+TLS+JSON. |
| Preconditions | Controller and router R1 are running. TLS connection is established. |
| Input data | Routing table for R1 with at least 3 entries. |
| Test steps | 1. Generate routing table in controller. 2. Send UPDATE_TABLE message to R1: {"type": "UPDATE_TABLE", "entries": [...]}. 3. Check that R1 receives the message. 4. Verify table is stored in R1 local memory or lightweight local persistence. |
| Expected result | Router R1 receives and stores its routing table correctly in its local database. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Verifies both TCP delivery and local database storage. Automated after first execution. |

## TC-06: Display routing table in CLI

| Field | Description |
| --- | --- |
| Test case ID | TC-06 |
| Related requirement | FR-07 |
| Test name | Display routing table in CLI |
| Objective | Verify that the router displays the routing table in the terminal. |
| Preconditions | Router R1 has already received and stored a routing table. |
| Input data | Command: show routing-table |
| Test steps | 1. Start router R1. 2. Receive routing table from controller. 3. Execute display command in router CLI. 4. Verify terminal output format. |
| Expected result | The routing table is displayed with columns: Destination, Prefix Length, Next Hop, Cost. All entries are shown. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Terminal output is captured and compared against expected format in automated tests. |

## TC-07: Update link cost and recalculate routes

| Field | Description |
| --- | --- |
| Test case ID | TC-07 |
| Related requirement | FR-08 |
| Test name | Update link cost and recalculate routes |
| Objective | Verify that the controller recalculates and redistributes affected routes after a link cost change. |
| Preconditions | Initial topology and routing tables already exist and were distributed. |
| Input data | Change cost R1-R3 from 5 to 1. |
| Test steps | 1. Load initial topology (R1-R2 cost 2, R2-R3 cost 1, R1-R3 cost 5). 2. Generate and distribute initial routing tables. 3. Record current routing table for R1. 4. Change link cost R1-R3 to 1 from administrator CLI. 5. Controller recalculates Dijkstra and redistributes affected tables. 6. Compare old and new routing table for R1. |
| Expected result | Routing table for R1 is updated: path to R3 is now R1 -> R3 (cost 1) instead of R1 -> R2 -> R3 (cost 3). |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Verifies both recalculation and redistribution. Automated after first guided execution. |

## TC-08: Invalid JSON message handling

| Field | Description |
| --- | --- |
| Test case ID | TC-08 |
| Related requirement | NFR-05 |
| Test name | Invalid JSON message handling |
| Objective | Verify that the controller does not crash when it receives an invalid message. |
| Preconditions | Controller is running. |
| Input data | Malformed JSON message |
| Test steps | 1. Start controller. 2. Open a raw TCP connection to the controller. 3. Send malformed JSON string. 4. Observe controller behavior and logs. |
| Expected result | The controller rejects the message, logs an error with description, and continues running normally without crashing. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Also test with valid JSON but unknown message type. Both should be handled gracefully. Automated with unittest. |

TC-09: TLS encryption and token authentication

| Field | Description |
| --- | --- |
| Test case ID | TC-09 |
| Related requirement | FR-10, NFR-09 |
| Test name | TLS encryption and token authentication |
| Objective | Verify that all communication is encrypted with TLS and that routers without a valid token are rejected. |
| Preconditions | Controller is running with TLS configured. R1 has a valid token. A test client has an invalid token. |
| Input data | Valid token for R1. Invalid token: 'wrong_token_123'. |
| Test steps | 1. Start controller with TLS enabled. 2. Connect R1 with valid token. Verify AUTH_OK is received. 3. Connect a test client with invalid token. Verify AUTH_FAIL is received and connection is closed. 4. Verify successful TLS-protected connection using the configured SSL context and confirm that invalid-token clients are rejected. 5. Verify event is logged in controller database. |
| Expected result | R1 connects successfully. Invalid token client is rejected immediately. Message content is not readable without TLS decryption. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Token validation will be automated. TLS channel validation will be demonstrated through guided execution and connection behavior. |

TC-10: Heartbeat and fault detection

| Field | Description |
| --- | --- |
| Test case ID | TC-10 |
| Related requirement | FR-11, NFR-10 |
| Test name | Heartbeat and fault detection |
| Objective | Verify that the controller detects a router failure and automatically recalculates affected routes. |
| Preconditions | All 4-5 routers are running and registered. Routing tables have been distributed. |
| Input data | Router R4 is forcefully stopped (process killed). |
| Test steps | 1. Start all 5 routers and controller. 2. Verify routing tables are distributed correctly. 3. Forcefully stop router R4. 4. Wait for 3 missed heartbeat cycles according to the configured interval. 5. Verify controller marks R4 as 'caido' in database. 6. Verify controller recalculates Dijkstra excluding R4. 7. Verify affected routers receive updated routing tables. 8. Verify event is logged. |
| Expected result | After 3 missed heartbeats, R4 is marked as down. Routes that used R4 as next hop are recalculated and redistributed within acceptable time |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Automated after first guided execution. Timing may vary slightly by machine. |

TC-11: LPM packet forwarding simulation

| Field | Description |
| --- | --- |
| Test case ID | TC-11 |
| Related requirement | FR-05, FR-06 |
| Test name | LPM packet forwarding simulation |
| Objective | Verify that each router applies Longest Prefix Matching correctly to determine the next hop for a simulated packet. |
| Preconditions | Router R1 has received and stored its routing table. Table contains at least two entries with overlapping prefixes. |
| Input data | Destination IP: 192.168.1.200. Forwarding table entries: 192.168.0.0/16 -> R2, 192.168.1.0/24 -> R3. |
| Test steps | 1. Load forwarding table into R1 with overlapping prefix entries. 2. Send PACKET_SIM message to R1 with destination 192.168.1.200. 3. Verify R1 selects the /24 entry (more specific) over the /16 entry. 4. Verify next hop is R3. 5. Trace full hop-by-hop path from R1 to destination. |
| Expected result | R1 selects next hop R3 based on /24 prefix (longest match). Full hop-by-hop path is displayed correctly. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Core test of LPM logic. Automated as a pure unit test of the forwarding function. |

TC-12: FIFO queue and tail drop policy

| Field | Description |
| --- | --- |
| Test case ID | TC-12 |
| Related requirement | FR-12 |
| Test name | FIFO queue and tail drop policy |
| Objective | Verify that the router maintains a FIFO queue with a configurable maximum size and discards new packets when the queue is full. |
| Preconditions | Router R1 is running with queue max size configured to 5 packets. |
| Input data | 10 simulated packets sent in sequence to R1. |
| Test steps | 1. Configure R1 queue max size to 5. 2. Send 10 packets in rapid sequence to R1. 3. Verify that exactly 5 packets are queued. 4. Verify that 5 packets are discarded (tail drop). 5. Verify router queue state reflects 5 queued packets and 5 discarded packets, either in local persistence or runtime state.. |
| Expected result | First 5 packets are queued. Packets 6 to 10 are discarded. Database reflects correct states. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Automated after first guided execution. Queue size is configurable per router instance. |

TC-13: Metrics reporting to controller

| Field | Description |
| --- | --- |
| Test case ID | TC-13 |
| Related requirement | FR-13 |
| Test name | Metrics reporting to controller |
| Objective | Verify that each router sends periodic metrics to the controller and that the controller stores them correctly. |
| Preconditions | Controller and at least one router are running. Some packets have been processed. |
| Input data | Router R1 with queue size 3, 10 packets forwarded, 2 packets discarded. |
| Test steps | 1. Start controller and R1. 2. Process packets through R1 to generate metrics. 3. Wait for the configured metrics reporting cycle. 4. Verify R1 sends METRICS message: {"type": "METRICS", "router_id": "R1", "queue_size": 3, "forwarded": 10, "discarded": 2}. 5. Verify controller stores values in metricas_globales table with correct router_id and timestamp. |
| Expected result | Controller receives METRICS message from R1 and stores it correctly in metricas_globales. Values match the actual router state. |
| Actual result |  |
| Status |  |
| Evidence |  |
| Observations | Automated after first execution. For testing purposes, reporting interval can be reduced to 5 seconds. |

# 6. Requirements Traceability Matrix

| Requirement ID | Description | Test case ID | Test status |
| --- | --- | --- | --- |
| FR-01 | Router registration | TC-01 | Passed |
| FR-02 | Neighbor information exchange | TC-02 | Passed |
| FR-03 | Topology storage | TC-02 | Passed |
| FR-04 | Shortest path calculation (Dijkstra) | TC-03 | Pending |
| FR-05 | Routing table generation | TC-04, TC-11 | Pending |
| FR-06 | Routing table delivery and LPM forwarding | TC-05, TC-11 | Pending |
| FR-07 | Routing table display in CLI | TC-06 | Pending |
| FR-08 | Link cost update and route recalculation | TC-07 | Pending |
| FR-09 | Event logging in controller database | TC-09, TC-10 | Pending |
| FR-10 | TLS encryption of all communications | TC-09 | Pending |
| FR-11 | Heartbeat and automatic fault detection | TC-10 | Pending |
| FR-12 | FIFO queue with tail drop policy | TC-12 | Pending |
| FR-13 | Periodic metrics reporting to controller | TC-13 | Pending |
| NFR-05 | Invalid message handling | TC-08 | Pending |
| NFR-09 | TLS security — RSA handshake + AES session | TC-09 | Pending |
| NFR-10 | Availability — heartbeat fault detection | TC-10 | Pending |

# 7. Test Execution Summary

| Total test cases | Passed | Failed | Pending | Success percentage |
| --- | --- | --- | --- | --- |
| 13 | 2 | 0 | 11 | 15.38% |

# 8. Final Test Report

Write a short paragraph describing the general result of the testing process, the main issues found, and the improvements required before final delivery.

| Issue ID | Description | Severity | Related requirement | Suggested correction |
| --- | --- | --- | --- | --- |
| ISS-01 | No blocking issues remained in the validated Sprint 1 flow. Open items correspond to later-sprint functionality not yet implemented. | Low | Sprint 2 / Sprint 3 pending requirements | Keep current Sprint 1 baseline stable and continue with remaining backlog in the next sprint. |